# HDV1

# Description

The HDV1 System is a manual method providing the basic features necessary to comply with worldwide HDT/VICAT test specifications.  The HDV1 system can be configured for up to 6 stations, for multiple analysis within a single run.  The system includes a large, 16 liter insulated stainless steel bath with a powerful impeller stirrer system, offering superior thermal distribution within +/- 0.2° C.  The immersion heater allows for testing samples up to 300° C, and is controlled by a programmable temperature controller for rise rates of 50° C per hour or 2° C per minute.  The measurement of deflection is carried out with a dial indicator micrometer with a resolution of 0.01mm.

# Standard Features

Insulated Stainless Steel Bath

Immersion Heater, 300°C maximum Temperature

Programmable PID Temperature Control

-Selected Rise Rates of 50° C / Hour or 2° C / Minute

Specimen Test Frames Including:

Dial Indicator

ASTM 2° Thermometer

Variable Weight Set Includes :( 1 each - 1100, 500, 100, 50, 10, 5, 1 gm) Weights

(2 each - 200, 20, 2 gm) Weights = 2210 gm Total Each Station

HDT Foot

Vicat Adapter

Cooling Coil

Bath Drain